Segger J-Link

- Download and install J-Link Software and Documentation pack

  - https://www.segger.com/downloads/jlink/

nRF Command Line Tools

- Download and install nRF Command Line Tools

  - https://www.nordicsemi.com/Products/Development-tools/nRF-Command-Line-Tools/Download

nRF Utils

- Download and install nRF Utils

  - https://www.nordicsemi.com/Products/Development-tools/nRF-Util/Download#infotabs

- Create the following folder:

  - C:\nrf\utils

  - Move the downloaded nrfutil.exe into C:\nrf\utils

- Add C:\nrf\utils to your PATH in Environment Variables

- Open up command prompt

- Run the following command to install the SDK tools

  - nrfutil install nrf5sdk-tools

VS Code

- Download and install VS Code: https://code.visualstudio.com/.

nRF Connect

- Open VS Code.

- Go to the Extensions (left pane) view

- Search for nRF Connect for VS Code Extension Pack and install it.

  - When prompted, select Trust Publisher and Install

nRF Connect SDK and Toolchain:

- After installing the extension, a new nRF Connect icon on the left pane should appear.  Open the nRF Connect view.

  - You might be prompted about discontinued toolchains.  If so, select OK, do not show again.

- In the nRF Connect View, click on Install Toolchain

- Wait for tools to fetch available toolchains, then select v2.8.0.

  - This install will take some time.  Status will be shown in the bottom right corner.

SDK:

- After the toolchain is installed, back in the nRF Connect view, click on Manage SDK

- A dropdown will pop up, select Install SDK

- Choose v2.8.0, make sure the location is C:\ncs\v2.8.0 and hit enter to install the SDK.

  - This will also take some time.

Application:

- Go back to VS Code at the nRF Connect view and select Create a new application

  - When prompted, select Create a blank application

  - In the project location, enter C:\ncs\apps\cyber_scope, and hit enter.

  - A window will pop up indicating the new application has been created.  Click Open.

    - If prompted, check the Trust the authors box and click on Yes, I trust the authors.

- In file explorer, navigate to C:\ncs\apps\cyber_scope and check out the following from SVN:

  - https://maps110/svn/svnMAPS/trunk/projCyber/scope/sw

  - C:\ncs\apps\cyber_scope

Adding Boards:

- Next, in VS Code, go to File > Preferences > Settings.

  - In the search box, type board

  - Click on Add Item, enter C:\ncs\boards and click Ok.

- You can now close out of settings.

Building:

- In the nRF Connect view, click on + Add build configuration under Applications.

- Select the following, then click Generate and Build