# Shot data file format

The shot data file consists of two sections: header section and data section.  Both header and data sections will be stored in the statically allocated buffer in ram of size 64 KiB before being written to external flash memory.  The size of header section is fixed.  The actual size of shot data depends on some factors like gyroscope/accelerator ODR (output data rate), shot data duration, etc.  It must be less than the size of data section which is (64KiB – size of header section).  Figure 1 illustrates this idea.  In the future if we use an actual file system to store the shot data, this 64KiB size constraint can be removed and the file size can be less the 64KiB.

Figure 1.  shot data file format

# Header format

The header section is at the start of the file and is used to identify the file.  It consists of many fields which are described in the following table.  All the integer values are stored in little-endian format (i.e. least-significant byte at lowest address).

| Size (bytes) | Name | Description |
| --- | --- | --- |
| 4 | timeofShot | Timer value in tenth of second. |
| 1 | headerVer | Starting at 0. Different header versions will have different header formats. The phone should support all header version. |
| 1 | shotDataFormatVer | Starting at 0. |
| 4 | reserved | reserved. Can be used as time adjustment flags. |
| 4 | intensity | ambient light level when detecting a shot motion. |
| 4 | shotDataOffset | Offset in file where the shot data can be found. |
| 4 | shotDataSize | The size of shot data in bytes. |
| 1 | gyroBitsPerAxis | Number of bits of a sample per axis generated by gyroscope. Typical values are 8, 16, 24. |
| 1 | accelBitsPerAxis | Number of bits of a sample per axis generated by accelerometer. Typical values are 8, 16, 24. |
| 2 | gyroSampleRateHz | Sample rate of gyroscope in Hz. |
| 2 | accelSampleRateHz | Sample rate of accelerometer in Hz. |
| 1 | gyroFullScale | Unit: dps (degrees per second) |
| 2 | accelFullScale | Unit: g |
| 2 | IMUOrientation | See Table 2 for bit map and Table 3 for nibble definition. |

Table 1.  Header format specification

| unused | away from user | to the right of user | towards earth |
| --- | --- | --- | --- |
| 0000 | nibble 2 | nibble 1 | nibble 0 |

Table 2.  bit map for IMUOrientation field

| 0111 | unused |
| --- | --- |
| … | unused |
| 0100 | unused |
| 0011 | +z |
| 0010 | +y |
| 0001 | +x |
| 0000 | unused |
| 1111 | -x |
| 1110 | -y |
| 1101 | -z |
| 1100 | unused |
| … | unused |
| 1000 | unused |

Table 3.  nimble definition

# Shot data format type 0

The shot data has a repeating pattern which changes depending on the output data rates and bits per sample per axis.  The examples below show how to determine the shot data format based on different ODRs.  Each sample is stored in little-endian format (i.e. least-significant byte at lowest address).

## Example 1

| gyroODRHz = 416 accelODRHz = 416 |  |
| --- | --- |
| Time | Sample |
| t0 | Gx |
| t0 | Gy |
| t0 | Gz |
| t0 | Ax |
| t0 | Ay |
| t0 | Az |
| repeat… |  |

Table 4.  shot data format example 1

## Example 2

| gyroODRHz = 416 accelODRHz = 208 |  |
| --- | --- |
| Time | Sample |
| t0 | Gx |
| t0 | Gy |
| t0 | Gz |
| t0 | Ax |
| t0 | Ay |
| t0 | Az |
| t1 | Gx |
| t1 | Gy |
| t1 | Gz |
| repeat… |  |

Table 5.  shot data format example 2

## Example 3

| gyroODRHz = 104 accelODRHz = 416 |  |
| --- | --- |
| Time | Sample |
| t0 | Gx |
| t0 | Gy |
| t0 | Gz |
| t0 | Ax |
| t0 | Ay |
| t0 | Az |
| t1 | Ax |
| t1 | Ay |
| t1 | Az |
| t2 | Ax |
| t2 | Ay |
| t2 | Az |
| t3 | Ax |
| t3 | Ay |
| t3 | Az |
| repeat… |  |

Table 6.  shot data format example 3